2/4 Port USB KVM Switch

Thanks for buying our USB KVM switch. It allows you to access two or four computers /DVR/NVR from a single USB keyboard,USB mouse and monitor console. They are widely used in security exchange market, multimedia conference room,booking hall ect...

◆Product Features:

• One USB console controls 2/4 VGA-interface computers /DVR/NVR

• Switch Mode: a. button push  b. Keyboard Hot-Key switch

• With stereo audio and microphone

• Superior video quality :2048×1536@60Hz

• Multiplatform support: Windows 10/7/2000/XP/Vista、Linux and Mac;

• Support Hot plug & play

• Support auto detection

• Support auto Scan Mode for monitoring all computers /DVR/NVR

• With USB2.0 Hub

◆Switch Modes(For 2port kvm switch)：

• Button Switch：Switch to any port by buttons on front panel.

• Keyboard Hot-key Switch : Double click the Hot-Key on the keyboard, just like the right【Ctrl】

◆Hotkey Setting(For 2port kvm switch)：

Default hot-key is the right【Ctrl】.If use the right【Ctrl】conflict with other hot-key，The hot-key can be set to【Num lock】 key，or【Scroll Lock】.

For example：Double click the right【Ctrl】+【Num lock】，The hot-key change to 【Num lock】；double click the right【Ctrl】+【Scroll Lock】，The hot-key change to 【Scroll Lock】key.If the hot-key set as 【Scroll Lock】，and you want to set the hot-key as the left【Ctrl】 key，then double click the【Scroll Lock】 +the left【Ctrl】，then the hot-key change to the left 【Ctrl】.

| Double click the right【Ctrl】+【Num lock】 | Hotkey change to【Num lock】 |
| --- | --- |
| Double click the right【Ctrl】+left【Ctrl】 | Hotkey change to th left【Ctrl】 |
| Double click the right【Ctrl】+【Scroll Lock】 | Hotkey change to 【Scroll Lock】 |
| If the hotkey is changed to 【Scroll Lock】，You want to change it to the left 【Ctrl】 ，Then double click 【Scroll Lock】 +the left【Ctrl】 |  |

◆Keyboard hot-key Operation(For 2port kvm switch)：

| Double click the right【Ctrl】+【S】 | Turn on/off the Anto-scan function,You aslo can press the scan on front panel |
| --- | --- |
| Double click the right【Ctrl】+【S】+n | Set the scan interval，Can set to 8-999 seconds,the n stands for Number 8 to 999.（The system defaults to 8 seconds） |
| Double click the right【Ctrl】+【T】 | Turn on/off the detection function (default is off) |
| Double click the right【Ctrl】+【R】 | Reset the KVM System to solve the keyboard and mouse freeze. |

--------------------------------------------------------------------------------------------------------------------------------------------------------

◆Switch Modes(For 4port kvm switch)：

• Button Switch：Switch to any port by buttons on front panel.

• Keyboard Hot-key Switch : Double click the right【Ctrl】 +1/2/3/4【Enter】

◆Hotkey Setting(For 4port kvm switch)：

Default hot-key is the right【Ctrl】.If use the right【Ctrl】conflict with other hot-key，The hot-key can be set to【Num lock】 key，or【Scroll Lock】.

For example：Double click the right【Ctrl】+【Num lock】+【Enter】，The hot-key change to 【Num lock】；double click the right【Ctrl】+【Scroll Lock】+【Enter】，The hot-key change to 【Scroll Lock】key.If the hot-key set as 【Scroll Lock】，and you want to set the hot-key as the left【Ctrl】 key，then double click the【Scroll Lock】 +the left【Ctrl】+【Enter】，then the hot-key change to the left 【Ctrl】.

| Double click the right【Ctrl】+【Num lock】+【Enter】 | Hotkey change to【Num lock】 |
| --- | --- |
| Double click the right【Ctrl】+left【Ctrl】+【Enter】 | Hotkey change to th left【Ctrl】 |
| Double click the right【Ctrl】+【Scroll Lock】+【Enter】 | Hotkey change to 【Scroll Lock】 |
| If the hotkey is changed to 【Scroll Lock】，You want to change it to the left【Ctrl】 ，Then double click 【Scroll Lock】 +the left【Ctrl】+【Enter】 |  |

◆Keyboard hot-key Operation(For 4port kvm switch)：

| Double click the right【Ctrl】+ 1/2/3/4+【Enter】 | The KVM can switch to the equipment you choose. |
| --- | --- |
| Double click the right【Ctrl】+【S】+【Enter】 | Turn on/off the Anto-scan function,You aslo can press the scan on front panel |
| Double click the right【Ctrl】+【S】+n+【Enter】 | Set the scan interval，Can set to 8-999 seconds,the n stands for Number 8 to 999.（The system defaults to 8 seconds） |
| Double click the right【Ctrl】+【T】+【Enter】 | Turn on/off the detection function (default is off) |
| Double click the right【Ctrl】+【R】+【Enter】 | Reset the KVM System to solve the keyboard and mouse freeze. |

◆Notes:

- When you connect the mouse and keyboard, please take care of the mouse and keyboard mark on the USB port. If you use a wireless keyboard and mouse kit, please insert the Bluetooth receiver on the USB port marked with the keyboard.

- When the auto scan function is activated,you can press the any key on the panel,the auto scan function turns off  automatically.

- If the computer enters into sleep mode .You must to press the compter’s power button to wake it up.we recommend the users do not set the computers enter sleep mode.

- If only one computer/DVR/NVR power on, the USB Hub can not connect other USB device, for the power is not enough.

- Orange lightning :The current opreating port.

◆Troubleshooting

Q: keyboard or mouse does not work

A: 1. first, make sure that the USB cable or socket are already connected to the computer side, there is no signal exception, and the power has been connected.

2. After the image switch normally, Mouse and keyboard access to the KVM switcher, wait a few seconds for the computer to complete the drive recognition.

3. Reset the KVM System to solve the keyboard and mouse freeze, Double click the right【Ctrl】+【R】

Q: No display

A: Check the cables if connected well or the cable problem.

Connection Diagram:

72UA

74UA